# HC-05 Bluetooth Module Change’s Guide

This document explains how to change the baud rate of an HC-05 Bluetooth module from 9600 to 38400 using Arduino UNO and Arduino IDE change its name and password .

## STEP 1 — Wiring Connection

HC-05 → Arduino UNO

VCC  → 5V
GND  → GND
TXD  → Pin 10
RXD  → Pin 11

## STEP 2 — Enter AT Mode

1. Hold the small button on the HC-05 module.
2. Power ON the module.
3. The LED should blink slowly.

Slow blinking means HC-05 is now in AT Mode.

## STEP 3 — Upload the Arduino Code

Upload the following code using Arduino IDE:

// Include SoftwareSerial library

// This library allows us to create another serial communication port

// because Arduino UNO has only one hardware serial port

#include <SoftwareSerial.h>

// Create a SoftwareSerial object named BT

// BT(10, 11)

//

// 10 = RX pin of Arduino

// 11 = TX pin of Arduino

//

// This means:

// Arduino pin 10 receives data

// Arduino pin 11 sends data

//

// Connection usually becomes:

//

// HC-05 TX  -> Arduino 10

// HC-05 RX  -> Arduino 11

SoftwareSerial BT(10, 11);

// setup() runs only ONE time when Arduino starts

void setup()

{

// Start normal Serial communication

// between Arduino and Computer

//

// 9600 is the baud rate

//

// This is used for:

// - Serial Monitor

// - Debugging

// - Seeing messages on screen

Serial.begin(9600);

// Start Bluetooth serial communication

//

// HC-05 AT mode usually works at 38400 baud rate

//

// IMPORTANT:

// This is NOT normal Bluetooth mode

// This is AT Command Mode

BT.begin(38400);

// Print text on Serial Monitor

//

// This helps us know that

// the system started successfully

Serial.println("AT Mode Ready");

}

// loop() runs again and again forever

void loop()

{

// Check if Bluetooth module has sent any data

//

// BT.available() returns:

// TRUE  -> if data is available

// FALSE -> if no data is available

if (BT.available())

{

// Read ONE character coming from Bluetooth module

//

// Example:

// HC-05 sends 'O'

//

// BT.read() reads that character

//

// Serial.write() immediately sends that character

// to the Serial Monitor

//

// This allows us to see HC-05 responses

// like:

// OK

// ERROR

// etc.

Serial.write(BT.read());

}

// Check if computer/Serial Monitor has sent data

//

// Example:

// You type:

// AT

//

// in Serial Monitor

if (Serial.available())

{

// Read the character typed in Serial Monitor

//

// Then send it to HC-05 module

//

// This allows us to send AT commands

// from computer to Bluetooth module

//

// Example:

// AT

// AT+UART=38400,0,0

// AT+NAME=RobotCar

BT.write(Serial.read());

}

}

## STEP 4 — Open Serial Monitor

Open Serial Monitor in Arduino IDE.

Set:
• Baud Rate = 9600
• Line Ending = Both NL & CR

## STEP 5 — Test AT Mode

Type:
AT

Expected Response:
OK

## STEP 6 — Change Baud Rate

Type:
AT+UART=38400,0,0

Expected Response:
OK

The baud rate is now changed to 38400.

## STEP 7 — Use the New Baud Rate

Now in your robot code:

BT.begin(38400);

will work.

To Change Back to 9600

Send:

AT+UART=9600,0,0

## Arduino Code for AT Mode

#include <SoftwareSerial.h>

SoftwareSerial BT(10, 11);

void setup()
{
  Serial.begin(9600);
  BT.begin(38400);

  Serial.println("AT Mode Ready");
}

void loop()
{
  if (BT.available())
  {
    Serial.write(BT.read());
  }

  if (Serial.available())
  {
    BT.write(Serial.read());
  }
}

## Useful HC-05 AT Commands

| Command | Purpose |
| --- | --- |
| AT | Test module |
| AT+UART=38400,0,0 | Set baud rate to 38400 |
| AT+UART=9600,0,0 | Set baud rate to 9600 |
| AT+NAME=RobotCar | Change Bluetooth name |
| AT+PSWD=1234 | Change Bluetooth password supports only 4 digits password |