This is the code to check whether the Arduino is working or not this code make the in-built led on the Audino blink for one second and stops for 1 second

STEP 1 — Check Whether Arduino UNO is Working

What this code does

- Turns ON the inbuilt LED for 1 second

- Turns OFF the LED for 1 second

- Repeats forever

Connections Required

No extra wiring needed.

Just connect:

- Arduino Uno → USB Cable → Computer

Steps to Execute

Step 1

Open Arduino IDE.

Step 2

Connect Arduino UNO using USB cable.

Step 3

Go to:

Tools → Board → Arduino AVR Boards → Arduino Uno

Step 4

Go to:

Tools → Port

Select COM port of Arduino.

Example:

COM7

To know the port search for device manager in your pc and search for ports in that from there see the port and then select the same in Arduino

Step 5

Paste Blink Code.

Step 6

Click Upload Button (→).

Result

If Arduino is working:

- Small orange LED marked “L” will blink continuously.

If Upload Error Comes

Check:

- USB cable

- COM port

- Board selection

void setup() {

// initialize digital pin LED_BUILTIN as an output.

pinMode(LED_BUILTIN, OUTPUT);

}

// the loop function runs over and over again forever

void loop() {

digitalWrite(LED_BUILTIN, HIGH);  // turn the LED on (HIGH is the voltage level)

delay(1000);                      // wait for a second

digitalWrite(LED_BUILTIN, LOW);   // turn the LED off by making the voltage LOW

delay(1000);                      // wait for a second

}

This is the code to check L293D motor driver shield so that we can check if all the motors are working or not .

Check L293D Motor Driver Shield

Connections Required

Attach L293D Shield

Place the shield properly on Arduino UNO.

Pins should align correctly.

Connect Motors

| Motor | Shield Port |
| --- | --- |
| Motor 1 | M1 |
| Motor 2 | M2 |
| Motor 3 | M3 |
| Motor 4 | M4 |

Power Supply

Option 1 (Small Motors)

USB can work.

Option 2 (Recommended)

Use external battery:

| Battery | Shield |
| --- | --- |
| + | EXT_PWR + |
| - | GND |

Example:

- 9V battery

- 12V battery pack

Install Library

Before uploading code:

Go to

Sketch → Include Library → Manage Libraries

Search:

AFMotor

Install:

Adafruit Motor Shield Library

Upload Steps

Step 1

Paste motor code from your file.

Step 2

Upload code.

Result

If everything works:

Motors will:

- Move Forward for 3 seconds

- Stop for 2 seconds

- Move Backward for 3 seconds

- Repeat

How to Check Connections

If Motor Not Moving

Check:

1. Shield Properly Inserted

All pins aligned.

2. External Power

Motor driver usually needs separate battery.

3. Motor Wires

Check both wires connected tightly.

4. Try Different Ports

Move motor from:

M1 → M2

to test.

#include <AFMotor.h>

// Create motor objects

AF_DCMotor motor1(1); // M1

AF_DCMotor motor2(2); // M2

AF_DCMotor motor3(3); // M3

AF_DCMotor motor4(4); // M4

void setup()

{

Serial.begin(9600);

// Set speed for all motors (0 - 255)

motor1.setSpeed(200);

motor2.setSpeed(200);

motor3.setSpeed(200);

motor4.setSpeed(200);

Serial.println("All Motor Test Started");

}

void loop()

{

// FORWARD

Serial.println("FORWARD");

motor1.run(FORWARD);

motor2.run(FORWARD);

motor3.run(FORWARD);

motor4.run(FORWARD);

delay(3000);

// STOP

Serial.println("STOP");

motor1.run(RELEASE);

motor2.run(RELEASE);

motor3.run(RELEASE);

motor4.run(RELEASE);

delay(2000);

// BACKWARD

Serial.println("BACKWARD");

motor1.run(BACKWARD);

motor2.run(BACKWARD);

motor3.run(BACKWARD);

motor4.run(BACKWARD);

delay(3000);

// STOP

Serial.println("STOP");

motor1.run(RELEASE);

motor2.run(RELEASE);

motor3.run(RELEASE);

motor4.run(RELEASE);

delay(2000);

}

This code is to check whether the bt is working or not

Check Bluetooth Module (HC-05 )

Bluetooth Wiring Connections

HC-05 → Arduino UNO

| Bluetooth Module | Arduino UNO |
| --- | --- |
| VCC | 5V |
| GND | GND |
| TXD | Pin 10 |
| RXD | Pin 11 |

Important

Bluetooth TX goes to Arduino RX.

Bluetooth RX goes to Arduino TX.

Upload Bluetooth Code

Step 1

Disconnect Bluetooth wires first.

Especially:

- TXD

- RXD

Step 2

Upload Bluetooth code.

Step 3

After upload reconnect wires.

Open Serial Monitor

Go to:

Tools → Serial Monitor

Set:

| Setting | Value |
| --- | --- |
| Baud Rate | 9600 |
| Line Ending | Both NL & CR |

Bluetooth LED Indication

Fast Blinking

Bluetooth ON but NOT connected.

Slow Blinking

Bluetooth connected successfully.

Mobile App for Testing

Install app:

Serial Bluetooth Terminal

or

Bluetooth Electronics

Steps to Connect Mobile

Step 1

Turn ON phone Bluetooth.

Step 2

Search devices.

Step 3

Connect to:

HC-05

or

HC-06

Password

Usually:

1234

or

0000

Send Commands

Inside Serial Bluetooth Terminal app:

Type:

HELLO

Result

If Bluetooth works:

- “HELLO” appears in Arduino Serial Monitor.

Test Reverse Communication

Type in Arduino Serial Monitor:

TEST

Press Send.

Result

“TEST” appears in mobile app.

#include <SoftwareSerial.h>

SoftwareSerial BT(10, 11);

// RX, TX

void setup()

{

Serial.begin(9600);

BT.begin(9600);

Serial.println("Bluetooth Test Started");

}

void loop()

{

// Data from Bluetooth to Serial Monitor

if (BT.available())

{

char data = BT.read();

Serial.write(data);

}

// Data from Serial Monitor to Bluetooth

if (Serial.available())

{

char data = Serial.read();

BT.write(data);

}

}